COURSE PROJECT REVIEW 1

BACSE324 | Embedded Systems Architecture

Submission Template

# Project Title

VisionGuard: Smart Cane for Visually Impaired People with Ultrasonic Obstacle Detection and Wet-Surface Warning

| Student name(s) | S Darshan – 25BCE2883 |
| --- | --- |
|  | Tanay Agarwal – 25BCE2475 |
| Programme / Section | B.Tech Computer Science and Engineering – Slot E2+TE2 |
| Faculty / Guide | Dr. Shabeeb Ahmed |
| Submission date | 27 October 2026 |

# 1. Abstract (100 words)

Visually impaired people depend on the white cane, which detects obstacles only on contact and cannot warn about water, puddles or open drains. VisionGuard is a low-cost smart cane built around an Arduino Nano (ATmega328P). An HC-SR04 ultrasonic sensor measures the distance to obstacles ahead, and a resistive moisture probe at the tip, read through the on-chip ADC, detects water or wet ground. Alerts are prioritised: a continuous buzzer tone for water, and beeps that get faster as an obstacle comes within 50 cm. Its distinguishing feature is combining obstacle and wet-surface sensing for under ₹900 in parts. Bench tests confirm obstacle alerts on battery power; threshold calibration and haptic feedback are in progress.

# 2. Circuit and Block diagram

Figure 1. Block diagram of VisionGuard

Figure 2. Circuit diagram. Pins as used in the firmware: Trig D5, Echo D6, moisture A0, buzzer D2; battery feeds the 5V pin through the rocker switch.

# 3. List of Components

List the main hardware and electronic components used in the prototype. Add or remove rows as required.

| S. No. | Component | Specification / Model | Function in the project |
| --- | --- | --- | --- |
| 1 | Arduino Nano | ATmega328P, 16 MHz, 5 V logic, 10-bit ADC | Main controller: reads both sensors, runs the alert logic, drives the buzzer |
| 2 | Ultrasonic sensor | HC-SR04, 40 kHz, 2–400 cm range | Measures distance to obstacles in front of the cane (Trig D5, Echo D6) |
| 3 | Moisture probe | Fork-type resistive probe | Mounted at the cane tip; detects water or wet mud on contact |
| 4 | Moisture signal module | LM393 comparator board (A0 / D0 outputs) | Converts probe resistance into an analog voltage read on A0 |
| 5 | Buzzer | Active buzzer, 5 V | Audio alert on D2: beeps for obstacles, continuous tone for water |
| 6 | Battery | 18650 Li-ion cell, 3.7 V, 1200 mAh | Portable power source |
| 7 | Battery holder | 18650 single-cell holder | Holds the cell on the cane |
| 8 | Rocker switch | SPST, 6 A | Power ON/OFF between battery and the 5V pin |
| 9 | Electrolytic capacitor | 220 µF / 470 µF | Smooths the supply during buzzer switching |
| 10 | Cane body and wiring | PVC pipe, jumper wires, berg strips, single-strand wire | Mechanical structure and electrical interconnections |

# 4. Bill of Materials (BOM)

Provide the estimated or actual cost of the components. Use the same currency throughout.

| S. No. | Item / Component | Qty. | Unit cost (₹) | Total cost (₹) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Arduino Nano (ATmega328P) | 1 | 250.00 | 250.00 | Vellore Robo Electronics |
| 2 | HC-SR04 ultrasonic sensor | 1 | 105.00 | 105.00 | Vellore Robo Electronics |
| 3 | Soil-moisture sensor (probe + LM393 module) | 1 | 45.00 | 45.00 | Vellore Robo Electronics |
| 4 | 18650 Li-ion cell, 3.7 V 1200 mAh | 2 | 65.00 | 130.00 | One fitted, one spare |
| 5 | 18650 cell holder | 1 | 30.00 | 30.00 | Billed as 2-cell; 1-cell holder fitted |
| 6 | Rocker switch, 6 A | 1 | 15.00 | 15.00 | Vellore Robo Electronics |
| 7 | Electrolytic capacitors (470 µF ×2, 220 µF ×2) | 4 | – | 45.00 | Supply filtering |
| 8 | Jumper wires (F-F, M-M, M-F; 10 each) | 3 sets | 20.00 | 60.00 | Vellore Robo Electronics |
| 9 | Berg strips (2 female, 3 male) | 5 | 10.00 | 50.00 | Vellore Robo Electronics |
| 10 | Single-strand wire | 3 | 12.00 | 36.00 | Vellore Robo Electronics |
| 11 | Nano USB cable | 1 | 40.00 | 40.00 | For programming |
| 12 | Active buzzer, 5 V | 1 | 20.00 | 20.00 | Estimated; not on bill |
| 13 | PVC pipe, hot glue, cable ties | – | – | 50.00 | Estimated; cane body |
| 14 | Tools: soldering iron, solder, flux, breadboard | 4 | – | 660.00 | Not part of the cane |
| 15 | Parts for planned features: IRLZ44N MOSFET, 1 A fuse + holder, JST connector, resistor box, 0.1 µF capacitors | – | – | 269.00 | For Review 2 upgrades |
| Estimated total project cost |  |  |  | 1,805.00 | Cane hardware (items 1–13): ₹876 |

Prices are from invoice Ref. 3293, Vellore Robo Electronics (07-10-2026), including 18% GST; the billed total is ₹1,735. Items 12–13 (₹70) are estimates.

# 4. Prototype Photograph (Working Hardware)

Insert a clear, recent photograph of the assembled working prototype below. The photograph should show the actual hardware (not only a simulation or circuit diagram). Ensure that the main components are visible; use a caption to briefly identify the prototype and, if useful, indicate its operating condition.

Figure 3. VisionGuard prototype on a PVC cane. Top: LM393 module, HC-SR04 sensor, Arduino Nano with buzzer and switch. Bottom left: moisture probe at the tip. Bottom right: 18650 cell in its holder. Powered ON, the prototype gives obstacle alerts on battery power.

# 5. Current Status at Review 1 and Potential improvements for final submission

Briefly indicate what has been completed and what remains.

Completed: problem study; concept refined from an initial smart-shoe design to a cane (the tip reaches water before the foot, and a shaft-mounted sensor is steadier); component selection; assembly of the cane on a PVC pipe (sensors, Arduino Nano, buzzer, battery and switch soldered and mounted); firmware for ultrasonic ranging, moisture sensing and prioritised alerts written and uploaded.

Demonstrated functionality / preliminary results: on battery power, the buzzer beeps when an obstacle comes within 50 cm, and the beep rate increases as it moves closer (verified by moving a hand or notebook toward the sensor). The water alert (continuous tone) is implemented; dry and wet probe readings are being recorded on the Serial Monitor to calibrate the threshold.

Challenges or limitations: a single 3.7 V cell feeds the 5V pin directly, with no regulation or low-battery warning; the HC-SR04’s narrow (~15°) cone misses head-level and thin obstacles and drop-offs; the probe detects water only on contact and corrodes over time; the buzzer is hard to hear in traffic; the 50 cm range gives little warning at walking speed.

Next steps before Review 2: calibrate the moisture threshold and extend the obstacle range to about 100 cm; add a vibration motor driven by the IRLZ44N MOSFET; add a 5 V boost converter, charging module and the 1 A fuse; average several readings and space pings at least 60 ms apart (per the HC-SR04 datasheet) to reduce false alerts; tidy the wiring into an enclosure and field-test the cane.